GUIDE

(please focus on the highlighted words since that will be the base of the calculation)

Materials Calculation: Floorplan (height of column is fix to 2.7m (standard))

Column footing:
deformed bar cut length = 1.15 − (2 × 0.075)

6 16mm + 6 16mm = 12.0m 16mm deformed bar

Column:

200 width × 400 mm depth

in standard of 2.7 meters long column with 40 mm of cement cover

Vertical Deformed bar: 4 16mm × 2.7432 m = 10.9728 m 16mm deformed bar

Horizontal Deformed Bar Stirrups:

200 − 2(40) = 120 mm

400 − 2(40) = 320 mm

2(120 + 320) = 880 mm = 0.88 m per tie of 10mm deformed bar

24pcs 10mm deformed bar stirrups × 0.88 m = 21.12 m 10mm deformed bar

10.9728 m 16mm deformed bar and 21.12 m 10mm deformed bar for column

Column cement with mixture of 1:2:4

0.20 × 0.40 × 2.7432 = 0.21946 m3

0.33797 × 1/7 = 0.04828 m3

0.04828 × 1440 = 69.52 kg

69.52/40 = 1.7 bags + 5% waste = 1.83 40kg bag of cement

Sand: 0.101 m³

Gravel: 0.203 m³

In Total per 2.7m Column with 5% waste:

16mm Deformed Bar: 10.97m

10mm deformed Bar: 21.12 m

Cement: 1.83 40kg bag of cement

Sand: 0.101 m³

Gravel: 0.203 m³

Beam

200 × 400 mm section and reinforcement standard

Beam width = 0.20 m

Beam Depth = 0.40 m

Concrete Cover = 40mm

Top Main Deformed Bars = 2 16mm Deformed Bar

Bottom Deformed Main Bars = 2 16mm deformed bar

16mm Deformed Bar = 4 x Length

Beam Stirrups: (hindi siya fixed so need calculation by the length of the beam)

Beam Strirrups Spacing: 50mm / 100mm / 150mm / 200mm

For example 5m of beam:

50mm: (0.50 / 0.05) + 1 = 11

100mm:  (0.50 / 0.10) + 1 = 6

150mm:  (1.00 / 0.15) + 1 = 8

200mm (Rest): 5 − 0.50 − 0.50 − 1.00 = 3.00m

= (3.00 / 0.20) + 1 = 16

= 11 + 6 + 8 + 16 = 41 beam stirrups for 5m length

Beam cement with mixture of 1:2:4 for a 5m beam

0.20 × 0.40 x L (Length)

mixture: 1 + 2 + 4 = 7

Concrete volume: 0.20 × 0.40 × 5 = 0.40m3

Dry Factor: 1m3 × 1.54 = 1.54m3

Cement portion: 1.54 × 1/7 ​= 0.22m3

approx cement density: 0.22 × 1440 = 316.8kg

= 316.8 / 40 = 7.92 bags​

in 5m beam with 5% waste: 0.40 × 7.92 = 3.3264 bags

Sand: 0.40 × 0.440 = 0.176 m3

with 5% waste: 0.176 × 1.05= 0.1848 m3

Gravel: 0.40 × 0.880 = 0.352 m3

with 5% waste: 0.352 ×1.05 = 0.3696m3

In Total per 5m of Beam with 5% waste:

16mm Deformed Bar: 20m

10mm deformed Bar: 36.08m

Cement: 3.33 bags

Sand: 0.1848 m³

Gravel: 0.370m3

Slab: (for example a 5m x 5m or 25m2 slab)

standards:

Slab thickness: 100mm

Concrete cover: 20mm

Reinforced Deformed bars: 10mm deformed bars

Spacing: 200mm

Distribution Deformed bars: 10mm deformed bars

Spacing: 200mm

Mixture type: 1:2:4

(I shortcut ko nalang yah, dami eh)

example: In 5m x 5m or 25m2 of slab with 5% waste:

10mm deformed bar: 210m

Concrete: 19.8 bag

Sand: 1.16 m³

Gravel: 2.31m3

Wall (Interior and Exterior) mixture type: 1:3

For example of 5m wall in length and height of 2.7m height

Walls (Interior and Exterior since CHB height and Long is identical)

Exterior walls = 6inch 3 core CHB

Interior Walls = 4inch 3 core CHB

CHB length = 5.00 − 0.30 − 0.30 = 4.40m

wall area: 5.00 × 2.70 = 13.50m2​

CHB coverage: 0.40×0.20 = 0.08m2

13.50 ÷ 0.08 = 168.75 CHB

+ 5% waste = 168.75×1.05= 177.1875

for a 5m wall in length and height of 2.7m height:

CHB: 178 pcs

Concrete: 2.86pcs 40kg bag

Sand: 0.11m3